| Q | Scheme | Marks | AOs | Pearson Progression Step and Progress descriptor |
| --- | --- | --- | --- | --- |
| 1a | Force = 4 × 9.8 = 39.2 (N). Accept 39. | M1 | 1.1b | 4th Calculate moments. |
|  | Moment = force × distance | M1 | 1.1a |  |
|  | Moment = 39.2 × 3 = 117.6 (N m). Accept 118. | A1 | 1.1b |  |
|  |  | (3) |  |  |
| 1b | Moment = F × 7 = 7F (N m) | A1 | 1.1b | 4th Calculate moments. |
|  |  | (1) |  |  |
| 1c | Equal moments | M1 | 1.1a | 5th Calculate sums of moments. |
|  | Solve for F | M1 | 1.1b |  |
|  | 16.8 (N). Accept 17. | A1ft | 1.1b |  |
|  |  | (3) |  |  |
| (7 marks) |  |  |  |  |
| Notes |  |  |  |  |

| Q | Scheme | Marks | AOs | Pearson Progression Step and Progress descriptor |
| --- | --- | --- | --- | --- |
| 2a | Figure 1 Force labels one mark each Allow explicit evaluation with g. | B2 | 2.5 | 4th Calculate moments. |
|  |  | (2) |  |  |
| 2b | Alice: Moment = 2 × 50 × g | M1 | 1.1b | 5th Calculate sums of moments. |
|  | = 100g (N m) | A1 | 1.1b |  |
|  | Bob: Moment = (2 − x) × 80 × g | M1 | 3.4 |  |
|  | = 80(2 − x)g (N m) | A1 | 1.1b |  |
|  | Total clockwise moment = 20g(4x − 3) (N m) | A1 | 1.1b |  |
|  |  | (5) |  |  |
| 2c | Equating to 0 and solving | M1 | 3.4 | 5th Solve equilibrium problems involving horizontal bars. |
|  | x = 0.75 (m) | A1 | 1.1b |  |
|  |  | (2) |  |  |
| 2d | Identifying 2 as a limit | M1 | 2.4 | 7th Solve problems involving bodies on the point of tilting. |
|  | So tilts towards Alice when 0.75 < x ⩽ 2 | A1 | 2.2a |  |
|  |  | (2) |  |  |

| 2e | Any valid limitation. For example, Pivot not a point. Alice can’t sit exactly on the end. The see-saw might bend. | A1 | 3.5 | 3rd Understand assumptions common in mathematical modelling. |
| --- | --- | --- | --- | --- |
|  |  | (1) |  |  |
| (12 marks) |  |  |  |  |
| Notes 2d Allow any similar valid argument. |  |  |  |  |
| Q | Scheme | Marks | AOs | Pearson Progression Step and Progress descriptor |
| 3a | Moment from bus = 5000 × 2 × g | M1 | 3.1a | 5th Find resultant moments by considering direction. |
|  | = 10 000g (N m) | A1 | 1.1b |  |
|  | Moment from gold = 1000 × 12 × g | M1 | 3.1b |  |
|  | = 12 000g (N m) | A1 | 1.1b |  |
|  | Moment from people = 70 × 8 × n × g | M1 | 3.1a |  |
|  | = 560ng (N m) | A1 | 1.1b |  |
|  | Total moment = (22 000 − 560n)g (N m) | A1 | 1.1b |  |
|  |  | (7) |  |  |
| 3b | Forming an equation or inequality for n and solving to find (n = 39.28…) | M1 | 1.1b | 5th Solve equilibrium problems involving horizontal bars. |
|  | Need 40 people. | A1 | 3.2a |  |
|  |  | (2) |  |  |
| 3c | New moment from gold and extra person is 1070 × 12 × g (N) | M1 | 3.1a | 5th Solve equilibrium problems involving horizontal bars. |
|  | New total moment = (22840 − 560n)g (N m) | M1 | 1.1b |  |
|  | n = 40.78… | A1 | 3.2a |  |
|  | 42 people (including the extra) | A1 | 2.4 |  |
|  |  | (4) |  |  |
| (13 marks) |  |  |  |  |
| Notes Allow explicit calculations with g evaluated. |  |  |  |  |

| Q | Scheme | Marks | AOs | Pearson Progression Step and Progress descriptor |
| --- | --- | --- | --- | --- |
| 4 | Weight of right mass is 10g (N) | M1 | 1.1b | 7th Solve problems involving bodies on the point of tilting. |
|  | Moment on right gear is force × distance from centre | M1 | 3.1b |  |
|  | Moment = 10g × 0.08 = 0.8g (N m) | A1 | 1.1b |  |
|  | Force on left gear by right gear is | M1 | 3.1b |  |
|  | Force on left gear by right is= 8g | A1 | 1.1b |  |
|  | Moment on left gear is force × distance from centre | M1 | 3.1b |  |
|  | Moment = 8g × 0.05 = 0.4g (N m) | A1 | 1.1b |  |
|  | Weight = | M1 | 1.1a |  |
|  | Weight == 20g (N) | M1 | 1.1b |  |
|  | M = 20g ÷g = 20 (kg) | A1 | 1.1b |  |
| (10 marks) |  |  |  |  |
| Notes Allow calculations with g evaluated. |  |  |  |  |

| Q | Scheme | Marks | AOs | Pearson Progression Step and Progress descriptor |
| --- | --- | --- | --- | --- |
| 5 | Moment on see-saw is force × distance from pivot. | M1 | 1.1a | 5th Solve equilibrium problems involving horizontal bars. |
|  | Moment on Poppy’s see-saw due to Poppy is pg × 3 = 3pg (N m) | M1 | 2.2a |  |
|  | Force on Bob due to Poppy is (N) | A1 | 2.2a |  |
|  | Force on Bob due to Quentin is (N) | A1 | 2.2a |  |
|  | Total force on Bob is (N) | M1 | 2.2a |  |
|  | Weight of Bob is 80g (N) | M1 | 1.1b |  |
|  | Forces are equal so = 80g | M1 | 3.1b |  |
|  | p + q = 53 to the nearest whole number. | A1 | 2.4 |  |
| (8 marks) |  |  |  |  |
| Notes Allow calculations with g evaluated. |  |  |  |  |